The University of Texas at Austin

Dept. of Electrical and Computer Engineering

Midterm #2

Prof. Brian L. Evans

Date: December 6, 2013						Course: EE 445S

Name:

Last, 					First

- The exam is scheduled to last 50 minutes.

- Open books and open notes.  You may refer to your homework assignments and the homework solution sets.  You may not share materials with other students.

- Calculators are allowed.

- You may use any standalone computer system, i.e. one that is not connected to a network. Disable all wireless access from your standalone computer system.

- Please turn off all cell phones and other personal communication devices.

- All work should be performed on the quiz itself.  If more space is needed, then use the backs of the pages.

- Fully justify your answers unless instructed otherwise.  When justifying your answers, you may refer to the Johnson, Sethares & Klein textbook, the Welch, Wright and Morrow lab book, course reader, and course handouts.  Please be sure to reference the page/slide number and quote the particular content you are using in your justification.

| Problem | Point Value | Your score | Topic |
| --- | --- | --- | --- |
| 1 | 25 |  | Channel Equalization |
| 2 | 27 |  | Receiver Design |
| 3 | 30 |  | Pre-emphasis |
| 4 | 18 |  | Potpourri |
| Total | 100 |  |  |

Problem 2.1. Channel Equalization. 25 points.

In the discrete-time system on the right, the
equalizer operates at the sampling rate.

Equalizer is a finite impulse response (FIR) filter with two real coefficients w0 and w1:

r[k] = w0 y[k] + w1 y[k-1]

You may ignore the noise signal nk.

- For the adaptive FIR equalizer, derive the update equation for w1 for the following objective function:   12 points.

J(e[k])=14e4[k]

- One of the problems with the adaptive FIR equalizer in part (a) is that its convergence depends on the initial value for w1.

Consider finding the roots of the polynomial

Jx=14x4

i. Give the iterative update equation for estimates for x.   6 points.

ii. From the iterative update equation in part i, give the range of initial values of x to guarantee convergence.  The range of values may depend on the step size .  7 points

Problem 2.2  Receiver Design. 27 points.

Consider the baseband pulse amplitude modulation (PAM) receiver blocks below with

- sampling rate fs

- downsampling factor L = 6 samples/symbol where fs = L fsym

- square root raised cosine pulse shape g[m] with rolloff parameter  = 1:

- Why is placing the FIR equalizer immediately after the A/D converter inefficient?  Completing steps (b)-(d) below might help you here.  6 points.

- The first step to remove the inefficiency is to swap the order of the equalizer and matched filter.  How can this be justified?  6 points.

- Show in the discrete time domain that downsampling by 6 is the same as downsampling by 3 followed by downsampling by 2.  6 points.

- The second step to reduce the inefficiency is to exchange the FIR equalizer with the downsampling by 3.   9 points.

- How can this exchange be justified?

- What is the frequency band in Hz over which the FIR equalizer has to equalize?

Problem 2.3.  Pre-emphasis.  30 points.

Consider an upconverted baseband 2-PAM signal x(t) = s(t) cos(2  fc t) where 
s(t) is a baseband 2-PAM signal with

Constellation spacing		2 d

Symbol rate			fsym

Sampling rate			fs

Samples per symbol		L = 20

Rolloff factor			 = 1

and where

Carrier frequency		fc = 2 fsym

Transmission bandwidth	B = 2 fsym

The received signal is r(t) = x(t) + n(t) where n(t) is spectrally-flat Gaussian noise.

Here is the block diagram for pre-emphasis filtering where q is an integer and q > 1:

The nonlinearity raises the input to the qth power.

(a) Give the passband and stopband frequencies for bandpass filter (BPF) #1.  6 points.

- Pre-emphasis of carrier frequency fc.  12 points.

i. give all possible values for q

ii. which value of q would you use and why?

iii. give the center frequency for BPF #2

(c) Pre-emphasis of symbol clock fsym.  12 points.

i. give all possible values for q

ii. which value of q would you use and why?

iii. give the center frequency for BPF #2

Problem 2.4. Potpourri.  18 points.

Please determine whether the following claims are true or false.  If you believe the claim to be false, then provide a counterexample.  If you believe the claim to be true, then give supporting evidence that may include formulas and graphs as appropriate.  If you give a true or false answer without any justification, then you will be awarded zero points for that answer.  If you answer by simply rephrasing the claim, you will be awarded zero points for that answer.

(a) Applying a lowpass filter to a spectrally-flat Gaussian noise signal always produces an output signal with lower average noise power than that of the input signal. 6 points.

- In discrete time, an ideal channel can be modeled as

The input x[m] can always be exactly recovered by discarding the first  samples of y[m] and scaling each subsequent sample by 1/g.  6 points.

(c) For a synchronized quadrature amplitude modulation (QAM) receiver and an additive spectrally-flat Gaussian noise channel, the in-phase noise will always be statistically independent of the quadrature noise when measured at the input to the decision block.  6 points.