% In-Lecture Assignment #1 on Sep. 18, 2024.  Based on homework problem 1.2.

% Key takeaways: (1) Chirp signals are useful in localization, testing and training

% because they linearly sweep a range of frequencies, and (2) Spectrograms analyze

% a signal in the time and frequency domains simultaneously so that frequencies can

% be localized in time. Spectrogram trades off frequency resolution for time resolution.

% Chirp Signals: Please see slides 1-14 to 1-16 of CommonSignalsInMatlab.pptx.

% Spectrograms:  Please see slides 1-17 to 1-20 of CommonSignalsInMatlab.pptx.

% Introduction: A chirp signal is a sinusoid whose principal frequency

% increases (or decreases) over time.  A chirp signal has the form

% c(t) = cos( q(t) )  where q(t) = 2 p ( f0 + 0.5 fstep t ) t = 2 p f0 t + p fstep t2

% The principal frequency in Hz is f0 when t = 0 and then changes over time at a

% rate of fstep in units of Hz/s. The principal frequency of a sinusoid at a given

% point in time is called the instantaneous frequency, and it is defined as

% dq (t) / dt in units of rad/s.  dq (t) / dt = 2 p f0 + 2 p fstep t = 2 p (f0 + fstep t).

% We divide dq (t) / dt by 2p to obtain instantaneous frequency in Hz of f0 + fstep t.

% (a) Generate a chirp signal that lasts 10s with f0 = 20 Hz and fstep = 420 Hz/s.

% Use sampling rate fs of 44100 Hz.  The chirp will sweep through the principal

% frequencies of the keys on an 88-key piano.  Here’s Matlab code to get started.

%%% Generate a chirp signal with frequency increasing

%%% from f0 to (f0 + fstep time) over time seconds

time = 10;

f0 = 20;

fstep = 420;

fs = 44100;

Ts = 1 / fs;

t = 0 : Ts : time;

%%% Add code here to define the chirp signal y = cos( angle(t) )

angle = 2*pi*f0*t + pi*fstep*t.^2;

y = cos(angle);

% (b) Play the chirp signal as an audio signal. Describe what you hear.

% I hear a rising pitch over time.  Sounds like a slide whistle or a tsunami warning siren

% (rb.gy/18exl).  Note:  Some laptop playback systems cannot play frequencies below 200 Hz.

sound(y, fs);

pause(time+1);

% (c) Plot the spectrogram of the chirp signal and describe the visual representation.

% Spectrogram shows a yellow line that represents the principal frequency in the chirp

% signal.  The line goes from 20 Hz at time 0s to 4220 Hz at time 10s.  The spectrogram

% plot is on the next page.  See Appendix A for explanation of spectrogram arguments.

figure;

blockSize = 256; overlap = 128;

spectrogram(y, hamming(blockSize), overlap, blockSize, fs, 'yaxis');
title('(c) Spectrogram with block size 256 and overlap 128');

% (d) Give the code for the spectrogram that would improve the

% frequency resolution by a factor of two vs. part (c).

% The frequency resolution is what is possible from observing a signal for a block of

% N samples which lasts for N Ts seconds.  From homework problem 0.1, the frequency

% frequency resolution in Hz is the inverse of the observation time or 1 / (N Ts) = fs / N.

% Increase N to decrease (improve) frequency resolution.

% The yellow line in the spectrogram with N doubled is half the width vs. part (c).

% Please see the derivation of frequency resolution in Appendix B.

figure;

blockSize = 2*256; overlap = 128;

spectrogram(y, hamming(blockSize), overlap, blockSize, fs, 'yaxis');
title('(d) Spectrogram with block size 512 and overlap 256');

% (e) Give the code for the spectrogram that would improve the time resolution,

% i.e. localizing frequency components in time, by a factor of two vs. part (c).

% The time resolution means the ability to identify when a frequency component occurs

% in time.  In a block of N samples, we do not know when frequency components occur, and

% hence, our time resolution in seconds is N Ts.  We improve time resolution by reducing N.

figure;

blockSize = 256/2; overlap = blockSize/2;

spectrogram(y, hamming(blockSize), overlap, blockSize, fs, 'yaxis');
title('(e) Spectrogram with block size 128 and overlap 64');

(e)

(d)

In all three spectrogram plots, the extent of the horizontal time axis is the same (from 0 to 10s) and the extent of the vertical frequency axis is the same (from 0 to ½ fs where fs = 44100 Hz).  We have chosen fs to satisfy the sampling theorem fs > 2 fmax where fmax is the maximum frequency of interest (4220 Hz) and to be a standard audio sampling rate.

(c)

Appendix A: Arguments to the MATLAB spectrogram function by Dan Jacobellis

Appendix B: Derivation of Frequency Resolution

Frequency resolution of Δf Hz means two frequency components spaced Δf Hz apart can each be clearly identified by an algorithm, e.g. well separated in a plot of the frequency domain.

We’ll illustrate the concept of frequency resolution by revisiting homework problem 0.1.

Homework 0.1 concerned a sine signal c(t) lasting from 0s to 1s.  The mathematical expression is a two-sided sine signal multiplied by a rectangular pulse that lasts from 0s to 1s:

c(t) = sin(2  fc t) rect(t – ½)

The continuous-time Fourier transform of r(t) = rect(t – ½) is a sinc function times a phase shift

Rf=F rectt-12 =sincf e-jπf where  sincx=sin⁡(πx)πx and

Cf=j2e-jπ(f+fc)sincf+fc-j2e-jπ(f-fc)sincf-fc due to the modulation property.

Below are the plots of |Rf| on the left and |Cf| for fc=4 Hz on the right:

Δf

For a signal lasting 0s to 1s and containing sinusoids at frequencies 3 Hz and 4 Hz,

c2(t) = sin(2  f0 t) rect(t – ½) + sin(2  f1 t) rect(t – ½)

let’s see if we can resolve the two frequencies.  We’re looking for two peaks in the frequency domain plot that are well separated at 3 Hz and 4 Hz.  Between the peaks, the magnitude response should not be higher than the “sidelobes” at frequencies higher than 1 Hz in Rf.

Clean separation of 3 Hz and
4 Hz frequency components

Difficulty separating 3.2 Hz and 4 Hz frequency components

More generally, for a rectangular pulse of duration T seconds, the frequency resolution is 1/T.  The value of 1/T is also the null bandwidth.

In the course of computing the spectrogram, we apply a rectangular pulse to the discrete-time signal to extract a block of samples to compute their Fourier series coefficients using the fast Fourier transform.  Consider a discrete-time signal that is a two-sided sine signal and the first N samples are kept:

c[n] = sin(wc n) rect((n – N/2)/N)

Here, r[n] = rect((n – N/2)/N) which has amplitude 1 for n ϵ {0, 1, …., N-1} and 0 elsewhere. We can also write r[n] = u[n] – u[n-N] where u[n] is the unit step function. The discrete-time Fourier transform of r[n] is a periodic sinc function times a phase shift :

Rω=sin⁡Nω2sinω2periodic sincfunctione-jωN-1/2

where w is in units of rad/sample. The periodic sinc function is periodic in w with period 2p.  Here’s one period of |R(ω)| for N = 8 :

Δω

w = -pi : 0.001 : pi;

N = 8;

Rw = sin(N*w/2) ./ sin(w/2);

figure;

plot(w, abs(Rw), 'k','LineWidth',2 );

title( '|R(w)|' );

xlabel( 'w' );

ylim( [0, 9] );

This is the magnitude response of an averaging filter with 8 coefficients.  Please see the Designing Averaging Filters handout.

The first zero for the magnitude response in positive frequencies occurs at 2p / N.  This is the null bandwidth and also the frequency resolution Δω.

Let’s connect the frequency resolution in the discrete-time frequency domain to the continuous-time frequency domain:

Δω=2πN=2πΔffs  means that Δf=fsN