The University of Texas at Austin

Dept. of Electrical and Computer Engineering

Midterm #2

Prof. Brian L. Evans

Date: May 4, 2012						Course: EE 445S

Name:

Last, 					First

- The exam is scheduled to last 50 minutes.

- Open books and open notes.  You may refer to your homework assignments and the homework solution sets.  You may not share materials with other students.

- Calculators are allowed.

- You may use any stand-alone computer system, i.e. one that is not connected to a network. Disable all wireless access from your stand-alone computer system.

- Please turn off all cell phones, pagers, and personal digital assistants (PDAs).

- All work should be performed on the quiz itself.  If more space is needed, then use the backs of the pages.

- Fully justify your answers unless instructed otherwise.  When justifying your answers, you may refer to the Johnson, Sethares & Klein textbook, the Welch, Wright and Morrow lab book, course reader, and course handouts.  Please be sure to reference the page/slide number and quote the particular content you are using in your justification.

| Problem | Point Value | Your score | Topic |
| --- | --- | --- | --- |
| 1 | 27 |  | Quadrature Amplitude Modulation |
| 2 | 27 |  | Channel Estimation |
| 3 | 27 |  | Pulse Amplitude Modulation Receiver |
| 4 | 19 |  | Noise Shaping |
| Total | 100 |  |  |

I

Q

d

d

-d

-d

Problem 2.1  Quadrature Amplitude Modulation (QAM). 27 points.

A 4-level QAM constellation is shown on the right.

Assume that the symbol time is 1s.

Assume that the energy in the pulse shape is 1.

(a) On the 4-QAM constellation on the right, please specify an
encoding for each level that minimizes the number of bit errors
when a symbol error occurs.   6 points.

(b) Compute the average and peak transmitted power.  3 points.

(c) Draw decision regions at the receiver on the above constellation.  6 points.

(d) Based on your decision regions in (c), give the fastest algorithm possible to decode/quantize the estimated symbol amplitude in the receiver into a symbol of bits.  6 points.

(e) Based on the decision regions in (c), give a formula for the probability of symbol error.  6 points.

b[m]

+

w[m]

s[m]

r[m]

Problem 2.2. Channel Estimation. 27 points.

A sparse communication channel is modeled

as a linear time-invariant (LTI) finite impulse

response (FIR) comb filter b[m] plus additive

white Gaussian noise w[m].

b[m] = [m ]  + [m – ]   where   > 1

w[m] has zero mean and variance 2.

- Give an equation in the discrete-time domain for the received signal r[m] in terms of the transmitted signal s[m] and the channel model. 6 points.

- Give a training signal s[m] that would enable accurate estimation of  for  > 1.  How would you determine the length of the training signal?  6 points.

- Using your answer in (b), give an algorithm in the receiver to estimate .  6 points

- Give a formula for the impulse response or transfer function of a channel equalizer in the receiver to compensate for the frequency selectivity of the channel.  You may ignore the noise.  9 points.

Problem 2.3.  Pulse Amplitude Modulation (PAM) Receiver.  27 points.

h[m]

L

r[m]

y[n]

v[m]

For a discrete-time baseband PAM receiver

when the channel is modeled as additive
white Gaussian noise, the first two blocks are:

r[m] is the discrete-time received signal.

g[m] is the pulse shape used in the transmitter.

Ng is the number of symbol periods in the pulse shape.

fsym is the symbol rate.

Note that   v[m] = h[m] * r[m]   and   y[n] = v[L n]

(a) Give a formula for the causal impulse response h[m] that maximizes a measure of signal-to-noise ratio at y[n].  6 points.

(b) How many multiplication-accumulation operations per second are needed for the two blocks above?  6 points.

The above cascade can be efficiently implemented as a polyphase filter bank as follows:

h0[n]

h1[n]

hM-1[n]

r[Ln]

r[Ln+1]

r[Ln+(L-1)]

r[m]

y[n]

(c) Give a formula of h0[n] in terms of h[n].  Hint: Compare y[Ng] for the direct form with y[Ng] of the polyphase filter bank.  6 points.

(d) How many multiplication-accumulation operations per second are needed to implement the above polyphase filter bank?  9 points.

Problem 2.4. Noise Shaping.  19 points.

Here is a block diagram of a noise-shaping feedback coder used in data conversion.

+

_

_

+

e[m]

b[m]

x[m]

v[m]

h[m]

quantizer

h[m] is the impulse response of a linear time-invariant (LTI) finite impulse response (FIR) filter.

This problem asks you to analyze the noise shaping.

- Replace the quantizer with an additive noise source w[m], i.e. b[m] = v[m] + w[m], and derive the transfer function in the frequency domain from the noise source w[m] to the output b[m].   Assume that the input x[m] is zero.  10 points.

- If the frequency selectivity of h[m] were lowpass, what is the frequency selectivity of the noise transfer function?  Sketch example frequency responses for both to help justify your answers.
9 points.