Tune-Up Tuesday #5 for October 14, 2021

The tuneup is to solve homework problem 5.3(b) and verify the solution.

Intro.  A step function u[n] is a function that turns “on” at the origin and stays on, as plotted on the right.  Mathematically,

un=1n≥00n<0

Problem. A linear time-invariant (LTI) system gives the output plotted on the right when the input is x1n=u[n]. The output, also called the step response, is

y1n=δn+2 δn-1-δ[n-2]

We’ll model the unknown LTI system as a finite impulse response (FIR) filter with input signal xn and output signal

yn=h0 xn+h1 xn-1+h2 xn-2+hN-1 x[n-N-1]

From this information, compute the filter coefficients hn for n=0, 1, …, N-1 and manually verify that the step response of the FIR filter is y1[n].

Deconvolution.  We’ll use deconvolution to compute the filter coefficients.  We derive the time-domain deconvolution algorithm by evaluating the output at n=0:

y0= b0 x0+b1 x-1+b2 x-2+⋯+bN-1 x-(N-1)

For LTI systems, it is a necessary (but not sufficient) condition for the system to be “at rest”, which means that all initial conditions x-1, x-2, …, x-N-1 must be zero. Since we know x[n] and y[n], we have one equation and one unknown at n = 0:

y0=b0 x0

and we can compute
b0=y[0]x[0]

For this calculation to be valid, the first value of the test signal, x[0], cannot be zero.

The second output value is:  y1= b0 x1+b1 x0, and therefore, b1=y1- b0x[1]x[0].

The third output value is:  y2= b0 x2+b1 x1+b2 x0 and b2=y2- b0x2- b1x[1]x[0].

In general, bN=yN-i=0N-1bix[N-i]x[0].

The MATLAB script utdeconvolve.m implements this algorithm.

Part (a).  Give the vectors for x and y that you used when running utdeconvolve.m and the filter coefficients in vector b that the code computes.

| x | y | b |
| --- | --- | --- |
| [ 1 1 1 ] | [ 1 2 -1 ] | [ 1 1 -3 ] |
| [ 1 1 1 1 ] | [ 1 2 -1 0 ] | [ 1 1 -3 1] |
| [ 1 1 1 1 1 ] | [ 1 2 -1 0 0 ] | [ 1 1 -3 1 0 ] |
| [ 1 1 1 1 1 1 ] | [ 1 2 -1 0 0 0 ] | [ 1 1 -3 1 0 0 ] |

Part (b). Verify that the filter coefficients by using them in the difference equation for the LTI FIR filter

yn=h0 xn+h1 xn-1+h2 xn-2+hN-1 x[n-N-1]

given that the input signal is u[n] and the output signal is y1[n]

y1n=un+un-1-3 un-2+ un-3

This is a closed-form solution for y1n which gives the correct output values for y1[n] for all n.  For n≥0, the values are [ 1 2 -1 0 0 0 …. ].

Alternate solution for part (b). One could simply compute several values of y[n]:

yn=xn+ xn-1-3 xn-2+xn-3

y0=x0+ x-1-3 x-2+x-3=1+0+3∙0+0=1

y1=x1+ x0-3 x-1+x-2=1+1-3∙0+0=2

y2=x2+ x1-3 x0+x-1=1+1-3∙1+0=-1

y3=x3+ x2-3 x1+x0=1+1-3∙1+1=0

y4=x4+ x3-3 x2+x1=1+1-3∙1+1=0

and so forth.