Property of Time-Invariance (Shift-Invariance) for a System Under Observation

Prof. Brian L. Evans and Mr. Jaehong Moon

The University of Texas at Austin, bevans@ece.utexas.edu

September 20, 2018 and updated January 2, 2023, and January 23, 2023

When evaluating system properties, we treat a system as a closed box and analyze the relationships between input signals and their corresponding output signals.  This process assumes that after inputting a signal, we can return the system to its original state.

A continuous-time system with input signal xt and output signal yt is time-invariant (shift-invariant) if whenever the input signal is delayed by t0 seconds, then the output signal will always be delayed by t0 seconds as well for all real values of t0.

A way to visualize the time-invariance property is to show the equivalence between

That is, does yshiftedt=yt-t0 for all possible real constant values of t0?

One-sided infinite observation.  Let’s consider the system under observation for t≥0-.  Time 0- means a time of 0 seconds before occurrence of a Dirac delta occurring at the origin.  We can only observe xt for t≥0- and yt for t≥0-.  This means that we can only observe xt-t0 for t≥t0and yt-t0 for t≥t0.

Example.  Consider a delay system that delays the input by T seconds, and we can only observe the input signal and the output signal for t≥0-.

Conceptually, the delay block can be thought as a long wire that conducts electricity from the input to the output.  Assuming electrons travel at 2/3 the speed of light, the length of the wire would be (2/3) c T where c is the speed of light (3 x 108 m/s).  Such an implementation would be impractical, but nonetheless helpful in analyzing the system.

The first observed output value y0 would be due to the initial conditions in the delay system.  In fact, the first T seconds of the output would due solely to the initial conditions in the system.   For input xt and output yt, once the initial conditions have been output, yT=x(0). That is, it takes T seconds for an input value (voltage) to arrive at the output.

The initial conditions for the delay system consist of the voltage values at different points in the wire at t = 0. Let’s denote these voltage values v(t) for –T < t  0.  That is, v0 will be first, and v-T will be the last, value among the initial conditions to be output.  The spatial location for the voltage v(t) for –T < t  0 is (-2/3) c t meters from the output location.

Let x(t) = 0 for 0  t < T and 1 for t  T.  For input x(t), the output is

yt=v-tfor 0 ≤ t < Txt–Tfor t ≥ T=v-tfor 0≤t<T0for T≤ t<2T1for t≥2T

Let’s keep the same initial conditions, i.e. v(t) for –T < t  0, and the same definition for signal x(t). Now, we input x(t-t0) into the delay system

xt-t0=unobservedfor 0 ≤ t < t00for t0 ≤ t < T+t01for t ≥ T+t0

and the output is

yshiftedt=v-tunobservedfor 0 ≤ t < Tfor T≤ t < T+t001for T+t0≤ t<2T+t0for t ≥2 T+t0=v-tfor 0 ≤ t < Tunobservedfor T≤ t < T+t0yt-t0for t ≥T+t0

yshiftedt only equals yt-t0 for t≥ T+t0 because the initial conditions did not shift in time even though the input did.

Plots of the signals are given next followed by an analysis of initial conditions:

If the system were time-invariant, then yshiftt=yt-t0 for all real t0 and t≥0-.  This holds for for t ≥T+t0.  For 0 ≤ t < T+t0, all unobserved values and initial conditions would have to be equal to a constant value.