Time Invariance for an Integrator

Prof. Brian L. Evans and Mr. Jaehong Moon

Dept. of Electrical and Computer Engineering

The University of Texas at Austin

February 17, 2022, and updated January 2, 2023

When evaluating system properties, we treat a system as a closed box and analyze the relationships between input signals and their corresponding output signals.  This process assumes that after inputting a signal, we can return the system to its original state.

A continuous-time system with input signal x(t) and output signal y(t) is time-invariant (shift-invariant) if whenever the input signal is delayed by t0 seconds, then the output signal will always be delayed by t0 seconds as well for all real values of t0.

A way to visualize the time-invariance property is to show the equivalence between

That is, does yshiftedt=yt-t0 for all possible real constant values for t0?

Integrator.  An integrator captures the idea of conservation or storage [1].  “Capacitors can be modeled as integrators (capacitors are reservoirs for electric charge).” [1]  The integrator integrates the input signal x(t); i.e., the output of the integrator is

yt=-∞txλdλ

Two-sided infinite observation.  For this case, we observe the input and output signals for all time, i.e. -∞<t<∞. We can determine if the system is time-invariant as follows.  We delay the input signal xt-t0 and analyze the resulting output signal

yshiftedt=-∞txλ -t0 dλ

and see if it is equal to yt-t0.  We can use a change of variables ξ=λ -t0 which means that dξ=dλ, ξupper=t -t0 and ξlower=-∞ to obtain

yshiftedt=-∞t-t0xξ dξ= yt-t0

Conclusion: An integrator under two-sided observation in time is time-invariant.

One-sided infinite observation.  In this case, we observe the input and output signals of the integrator for t≥0-.  Time 0- means a time of 0 seconds before occurrence of a Dirac delta occurring at the origin.  We will only allow positive values of t0 so that the delay for the input and/or output signals will be at times being observed.

Under what conditions will the integrator be time-invariant? [2]

The integrator output for t≥0- is

yt=-∞txλdλ=-∞0-xλdλ+0-txλdλ=C0+0-txλdλ

(1)

The scalar constant C0 captures the result of the integration of the input signal over time not observed from -∞<t<0.  Delaying the output signal by t0 seconds gives

yt-t0=C0+0-t-t0xλ dλ

for t≥0-.  For 0-< t< t0 , the integral accesses values of the input signal x(t) from -t0<t<0 which we are not able to observe.

When delaying the input signal xt-t0, the resulting output signal for t≥0- is

yshiftedt=C0+0-txλ -t0 dλ 
That is, the integrator has its own time reference, and shifting the input or output signal in time does not affect the clock internal to the integrator.

To help compare yt-t0 and yshiftedt, we perform algebraic manipulations on

yshiftedt=C0+0-txλ -t0 dλ

(2)

We use a substitution of variables ξ=λ-t0 which means that dξ=dλ, ξupper=t-t0 and ξupper=-t0 to obtain

yshiftedt=C0+-t0t-t0xξ dξ

yshiftedt=C0+-t00-xξ dξ +0-t-t0xξ dξ

yshiftedt=C0+C1(t0)+0-t-t0xξ dξ

For (1) and (2) to be equal for t≥t0, C1(t0) has to be zero for all values of t0.  That is, when integrating xt with respect to t from -t0 to 0-, the answer is always 0 regardless of the value of t0.  The only way for this to happen is if xt has amplitude of zero for -t0<t<0-.  This means that as t0→∞, C0→0.

Alternately, as t0→∞, C1(t0)→C0.  In the limit, a necessary condition for (1) and (2) to be equal for t≥t0 is for C0=0 to solve C0=2 C0.

Conclusion: An integrator observed for t≥0- is time-invariant if the initial condition is 0.

References

[1]  Pedro Albertos and Iven Mareels, Feedback and Control for Everyone, Springer, 2010.

[2] Stack Exchange, “Consider the integrator and check for time invariance”, April 14, 2019.