[11:00-11:10] Review of tune-up exercise and review from last lecture

Plotting a sinusoid in MATLAB may not produce a smooth plot. Increasing the sampling rate can help. When the sampling rates are low, discrete-time plots of sinusoids can be hard to recognize as sinusoids.

A time shift of a sinusoid results in a phase shift. We can calculate the phase shift from the time shift exactly, and we can use phase unwrapping algorithms to estimate the time shift from the phase shift.

The time shift/phase shift of a wave propagating through space can be used for navigation and ranging, including radio navigation and ranging (RADAR), and sound navigation and ranging (SONAR).

[11:10-11:20] Sound frequency and wavelength

The lowest audible sound is about 20 Hz. However, it is very difficult to produce a pure 20 Hz sinusoid, since it would require a large and powerful subwoofer.

Wavelength (λ, in meters) is related to frequency (f, in Hertz) by the speed of the wave (c, in meters per second):

λ=cf

Speed of sound in air: 340 m/s

Speed of sound in water: 1500 m/s

Speed of light/radio waves: 3×108

To produce larger wavelengths, you need loudspeaker or antenna whose size is proportional to the wavelength. Example:

340 m/s20 Hz=17m

To reproduce a sampled signal, we require the sampling rate fs>2fmax →fmax <12fs

[11:20-] Sampling

Many signals are initially measured as continuous signals (e.g. sound converted from continuous pressure signal to a continuous voltage signal via transducer)

Sampling a continuous signal at equally spaced intervals Tn in time produces a discrete time signal:

yndiscrete time signal=y(nTs)continuous signal evaluated at integermultiples of the sampling period

Example: sampling a sinusoid:

yt=Acos2πf0t+ϕ

yn=yt​t=nTs=Acos2πf0Tsω0n+ϕ

ω0=2πf0Ts=2πf0fs  is the discrete time frequency in units of radians per sample.

How small does Ts need to be (or how high does fs need to be) to for an accurate plot?

[11:35] Exponential signals

Complex exponentials are related to sinusoids:

ejθ=cosθ+jsinθe-jθ=cosθ-jsinθ→cosθ=12ejθ+e-jθsinθ=12jejθ-e-jθ

[11:45-12:15] Signal properties and spectrum representation

Signals can be defined or described in many ways:

- As a function (e.g. cos(t))

- As a sequence of numbers

- By its properties (e.g. symmetric)

- As a piecewise combination of other signals

Signals can be represented spectrally as a combination of sinusoids:

xt=A0+k=1NAkcos2πfkt+ϕk=X0+Rek=1NXkej2πfkt

Where A0 is a real-valued and X0=A0 and Xk=Akejϕk

Example (slide 1-16)

xt=cos2π⋅440⋅t

xt=cosA0+A1cos2π⋅440⋅t+A1cos2π⋅880⋅t

| A0 | 0 |  |  |
| --- | --- | --- | --- |
| A1 | 1 | A2 | 0 |
| f1 | 440 Hz | f2 | 880 Hz |
| ϕ1 | 0 | ϕ2 | 0 |

yt=x2t=cos22π⋅440⋅t=12+12cos2π⋅880⋅t

| A0 | ½ |  |  |
| --- | --- | --- | --- |
| A1 | 1 | A2 | 0.5 |
| f1 | 440 Hz | f2 | 880 Hz |
| ϕ1 | 0 | ϕ2 | 0 |

Two sided spectrum:

xt=A0+k=1NAkcos2πfkt+ϕk=X0+k=1NXk2ej2πfkt+Xk*2e-j2πfkt

Where xk=Akejϕk and Xk*=Ake-jϕk

Example:

xt=10+14cos200πt-π3+8cos500πt-π2
=10+7e-jπ3ej2π100t+7ejπ3e-j2π100t+4ejπ2ej2π250t+4e-jπ2e-j2π250t

[12:15-] Beat Notes

When two sinusoids are multiplied, the result is two frequencies (sum and difference)

xt=cos2π10tcos2π1000t
=12cos2π 1010 t-π2+12cos2π 990 t-π2

Doc cam