Tune-Up Tuesday for September 7, 2021

% (a) Copy, paste and run the Matlab code from lecture slide 1-16 to generate
% the cosine signal x(t) = cos(2 f0 t) with f0 = 440 Hz and play it as an audio
% signal for 3s at a sampling rate of fs = 8000 Hz.  (Using a higher standard
% audio sampling rate such as 96000 Hz would give a smoother plot in part (d)).

% (a) ‘A’ note on Western scale in 4th octave (A4) @ 440 Hz

f0 = 440;               % note ‘A4’

fs = 8000;              % sampling rate

Ts = 1/fs;              % sampling time

t = 0 : Ts : 3;         % 3 seconds

x = cos(2*pi*f0*t);

sound(x, fs);
pause(3);

% (b) Add to the code in (a) to generate a new signal y(t) = cos(2 f0 t) cos(2 f1 t)
% with f1 = 110 Hz by using the same sampling rate of fs = 8000 Hz.

% (b) Multiply cosine @ 440 Hz and cosine @ 110 Hz

% Modified code from left side of lecture slide 3-3.

f1 = 110;

x1 = cos(2*pi*f1*t);

y = x .* x1;

% (c) Add to the code in (b) to playing y(t) as an audio signal.
%       Describe what you hear.
%       Express y(t) as a sum of two sinusoids.

% (c) Play y(t) at a sampling (playback) rate 8000 Hz.

%     I hear two notes/tones at a lower pitch than 440 Hz.

%     The product can be written as a sum of two cosines.

%     Using the result from lecture slide 3-2,

%     y(t) = 0.5 cos(2 pi 330 t) + 0.5 cos(2 pi 550 t)

%     Tones at 330 Hz (E4) and 550 Hz (C#5) are

%     harmonics of 110 Hz. See Piano frequencies.

sound(y, fs);

% (d) Plot one period of y(t).  We'll need to find the periodicity of y(t).
% The product of two sinusoids at frequencies f0 and f1 produces
% frequencies at f0+f1 and f0-f1.  You could modify the code from the
% bottom right side of lecture slide 3-3.

ffund = gcd(f0+f1, f0-f1); % 110 Hz which is note ‘A2’

Tfund = 1/ffund;

n = round(Tfund / Ts);     % Tfund / Ts isn’t an integer.

plot( t(1:n), y(1:n) );

% See the next page for the plot.

Non-smooth troughs