Tune-Up Tuesday for October 5, 2021

% Create a MATLAB simulation for fall 2021 midterm problem 1.2

% The problem involves analysis of the output of a squaring system y(t) = x2(t).

% Let input x(t) = cos(2 p f1 t) + cos(2 p f2 t) where f1 = 110 Hz and f2 = 220 Hz.

% (a) Using a sampling rate of 8000 Hz and time from 0 to 3 seconds, write

%        the code to generate a sampled version of the signal x(t).

fs = 8000;

Ts = 1/ fs;

tmax = 3;

t = 0 : Ts : tmax;

f1 = 110;

f2 = 220;

x = cos(2*pi*f1*t) + cos(2*pi*f2*t);

% (b) Play sampled version of x(t).  Describe what you hear.

% Answer:  The signal x(t) is composed of A note in the second octave (110 Hz)

% and A note in the third octave (220 Hz).

% On a laptop.  On many laptop speakers, the 110 Hz tone may not be audible

% due to limitations in playing back low audible frequencies.  On a laptop, the

% playback sounded like a single note with hum in the background.

% Audio system with a sub-woofer. A sub-woofer plays low audible frequencies

% down to 20 Hz.  The sub-woofer is often a separate large speaker (due to the

% longer acoustic wavelengths l for low frequencies, i.e. l = c / f) in an audio

% system.  On an audio system with a sub-woofer, the playback sounded like a

% beat frequency with hum in the background.  Both notes were audible.

soundsc(x, fs);

pause(tmax+1);

% (c) Plot the spectrum of the sampled version of x(t).  Principal frequencies

% are 110 Hz and 220 Hz.

% Using the spectrogram.  See the second page for the plot.

figure;

spectrogram(x, 512, 256, 512, fs, 'yaxis');

% Using the fast Fourier transform approach from mini-project #1.

fourierSeriesCoeffs = fft(x);

N = length(x);

freqResolution = fs / N;

ff = (-fs/2) : freqResolution : (fs/2)-freqResolution;

figure;

plot(ff, abs(fftshift(fourierSeriesCoeffs)));

xlabel('f');

xlim( [-1000, 1000] );

ylim( [-10, 15000] );

% (d) Using a sampling rate of 8000 Hz and time from 0 to 3 seconds,

%        write the code to generate the sampled version of the signal y(t).

y = x.^2;

% (e) Play sampled version of y(t). Describe what you hear.

% Answer:  The signal y(t) is composed of ‘A’ note in the second octave (110 Hz)

% ‘A’ note in the third octave (220 Hz), ‘E’ note in the fourth octave (330 Hz),

% and ‘A’ note in the fourth octave (440 Hz).  When played back, the signal

% y(t) has a higher pitch than x(t) but it was difficult to distinguish more than

% two notes.  The 0 Hz term is not audible.  Please see the answer in part (b).

soundsc(y, fs);

pause(tmax+1);

% (f) Plot the spectrum of the sampled version of x(t).  Principal frequencies

% are 0, 110, 220, 330, and 440 Hz.  Principal frequencies at 0, 330, 440 Hz aren’t

% in x(t) and are called intermodulation distortion caused by the squaring system.

% Using the spectrogram.  See below for the plot.

figure;

spectrogram(y, 512, 256, 512, fs, 'yaxis');

% Using the fast Fourier transform approach from mini-project #1.  See below.

fourierSeriesCoeffs = fft(y);

N = length(y);

freqResolution = fs / N;

ff = (-fs/2) : freqResolution : (fs/2)-freqResolution;

figure;

plot(ff, abs(fftshift(fourierSeriesCoeffs)));

xlabel('f');

Spectrogram for sampled x(t)

Spectrogram for sampled y(t)

xlim( [-1000, 1000] );

Spectrum for sampled y(t)

Spectrum for sampled x(t)