% Tune-Up #5                 October 10, 2023

% The tuneup is to solve homework problem 5.1.

% Intro.  A step function u[n] is a function

% that turns at the origin and stays on.  This

% can model turning on a switch and leaving it

% on indefinitely.  Mathematically, u[n] is

%       1 when n >= 0

%       0 otherwise.

% In Matlab, one can implement u[n] as ( n >= 0 ).

% The logical operator >= returns 1 if true and

% 0 if false.

% Part (a). Make a plot of u[n] for -5 <= n <= 10.

% Describe what you see.

n = -5 : 10;

unitstep = ( n >= 0 );

figure;

stem(n, unitstep);

xlabel('n');

ylabel('u[n]');

ylim([-0.5 1.5]);

% In the plot, the signal is zero/off when

% n < 0 and one/on when n >= 0.  This is a

% step function -- the signal takes a step up

% at n = 0 from amplitude 0 to amplitude 1.

% Part (b).  We can use the unit-step sequence

% to represent other sequences that are zero

% for n < 0.  Plot x[n] = (0.5)^n u[n]

% for -5 <= n <= 10. Describe what you see.

n = -5:10;

unitstep = ( n >= 0 );

x = (0.5 .^ n ) .* unitstep;

figure;

stem(n, x);

xlabel('n');

ylabel('0.5^n u[n]');

ylim([-0.5 1.5]);

% In the plot, signal is zero when n < 0 and a

% a decaying exponential sequence when n >= 0.

% Part (c).  Apply a four-point averaging

% filter to x[n] and plot the result

% for -5 <= n <= 10.  Describe what you see.

averagingFilterCoeffs = [ 1/4, 1/4, 1/4, 1/4 ];

y = filter(averagingFilterCoeffs, 1, x);

figure;

stem(n, y);

xlabel('n');

ylabel('filter output signal');

ylim([-0.5 1.5]);

% In the plot, the output signal is zero when

% n < 0.  The output signal for 0 <= n <= 2

% corresponds to a partial response by the

% filter to the change in the input signal at

% the origin.  Once we reach n = 3, the sliding

% window of input samples would be filled.