Tune-Up Tuesday #4 Deconvolution for October 10, 2024

Before we talk about deconvolution, let’s define convolution.  Then, we’ll derive a deconvolution algorithm and apply it to two examples.  You could have worked either example for the Tune-Up.

Convolution.  For a finite impulse response filter with M+1 filter coefficients b0, b1, …, bM, the output signal yn for an input signal xn is computed according to

yn=b0 xn+b1 xn-1+⋯+bM xn-M

If we input the discrete-time impulse signal δ[n], which has value of 1 at n=0 and 0 otherwise, the output is called the impulse response (response is synonymous with output):

hn=b0 δn+b1 δn-1+⋯+bM δn-M

Hence,

hk=bk  for k= 0, 1,…, M

Otherwise, hk=0.  Because hk=bk for k= 0, 1,…, M,

yn=h0xn+h1xn-1+⋯+hM+1xn-M+1=k=0Mhk x[n-k]

This computation is known as convolution of hn and xn:

yn=hn*x[n]

Given input signal xn and impulse response hn, we can compute output signal yn.  If xn is also finite in length, then the length of yn will be the length of hn plus the length of xn minus 1.

Deconvolution.  Whereas convolution computes the output signal yn from input signal xn and an impulse response hn of a FIR filter, deconvolution seeks to find impulse response hn given input signal xn and output signal yn.  We can choose the input signal xn, also known as a test signal, and observe the output signal.

Practical scenario.  We would start the test signal and the observation at a particular point in time, which we’ll say is at n=0 without loss of generality.  Further, we will assume that xn=0 for n<0; i.e., xn is a causal signal.

Deconvolution Algorithm.  We’ll work backwards in the time-domain to compute the FIR filter coefficients.  We derive a time-domain deconvolution algorithm by first evaluating the output at n=0:

y0=h[0] x0+h[1] x-1+h[2] x-2+⋯+h[M] x-M

As mentioned above, we’ll assume xn is a causal signal; i.e., xn=0 for n<0.  Since we know x[n] and y[n], we have one equation and one unknown at n = 0:

y0=h[0] x0

and we can compute
h[0]=y[0]x[0]

For this calculation to be valid, the first value of the test signal, x[0], cannot be zero.

Second output:  y1=h[0] x1+h[1] x0, and therefore, h[1]=y1- h0 x[1]x[0].

Third output:  y2=h[0] x2+h[1] x1+h[2] x0 and h[2]=y2- h0 x2-h1 x[1]x[0]

In general, for the Nth output,  h[N]=yN - i=0N-1hi x[N-i]x[0].

The MATLAB script utdeconvolve.m implements this algorithm.

Example #1.  Problem 4.3(b).  In this problem, we’re given

- causal input signal x[n] with non-zero values [ 1 2 3 4 5 ]

- causal output signal y[n] with non-zero values [ 1 1 1 1 1 -5 ]

We can compute the FIR filter coefficients using the above deconvolution algorithm:

h0=y0x0=11=1

h1=y1- h0 x1x0=1-1∙21=-1

h2=y2- h0 x2-h1 x1x0=1-1∙3-(-1)∙21=0

The values of h[n] for n>2 are zero.  The MATLAB script utdeconvolve.m will give the same answer for h[n].  We can validate the answer by convolving h[n] and x[n].  We can use the Matlab command conv to do this:

y = conv( [1 -1], [ 1 2 3 4 5 ] )

y =

1     1     1     1     1    -5

When convolving two finite-length signals xn and hn, the result yn has finite length.  The length of yn is the length of xn plus the number of filter coefficients minus 1.  Since the length of yn is 6 and the length of xn is 5, there are 2 filter coefficients.

Example #2.  Step response.  A step response is the response of a system to a step function.  A step function u[n] is a function that turns “on” at the origin and stays on, as plotted on the right.  Mathematically,

un=1n≥00n<0

When the input is x1n=u[n],  the step response for an FIR filter gives the output plotted on the right:

y1n=δn+2 δn-1-δ[n-2]

We can compute the FIR filter coefficients using the above deconvolution algorithm:

h0=y0x0=11=1

h1=y1- h0 x1x0=2-1∙11=1

h2=y2- h0 x2-h1 x1x0=-1-1∙1-1∙11=-3

h3=y3 - i=02hi x3-ix0=0-1∙1-1∙1-(-3)∙11=1

The values of h[n] for n>3 are zero.  We can validate the answer by convolving h[n] and x[n].  We can use the Matlab command filter to do this:

y = filter( [1 1 -3 1], 1, [ 1 1 1 1 1 1 1 1 ] )

y =

1     2    -1     0     0     0     0     0

We want to use the filter function because the input signal is infinite in length and we’ll have to truncate it to be of finite length in Matlab.  Convolving two finite length signals, e.g. using the conv function, will have trailing effects that would not be present for when the input signal is infinite in length.  The filter function also performs convolution but only computes as many output samples as there are input samples.

We can use utdeconvolve.m to compute the filter coefficients in vector b.

| x | y | b |
| --- | --- | --- |
| [ 1 1 1 ] | [ 1 2 -1 ] | [ 1 1 -3 ] |
| [ 1 1 1 1 ] | [ 1 2 -1 0 ] | [ 1 1 -3 1] |
| [ 1 1 1 1 1 ] | [ 1 2 -1 0 0 ] | [ 1 1 -3 1 0 ] |
| [ 1 1 1 1 1 1 ] | [ 1 2 -1 0 0 0 ] | [ 1 1 -3 1 0 0 ] |