% Tune-Up #6 - Fall 2025 - ECE 313 Linear Systems & Signals - Evans

% Copy this file into a Matlab script window, add your code and

% answers to the questions as Matlab comments, hit "Publish", and

% upload the resulting PDF file to this page for the tune-up

% assignment.  Please do not submit a link to a file but instead

% upload the file itself.   Late penalty: 2 points per minute late.

% Homework problem 6.1(a).

% For the first-order unnormalized averaging filter (lowpass filter)

%      y[n] = x[n] + x[n-1]

% and the initial condition x[-1] = 0

% as a necessary condition for LTI

% system properties to hold.

% Determine the formulas for, and

% plot in MATLAB, the following:

% i. impulse response

%    h[n] = d[n] + d[n-1] where d[n]

%    is the discrete-time impulse signal

n = [ 0 1 2 3 ];

filterCoeffs = [ 1 1 ];

impulse = ( n == 0 );

h = conv( filterCoeffs, impulse );

stem([n, 4], h, 'LineWidth',2 );

ylim( [-0.5 1.5] );

xlim( [-0.5 4.5 ] );

% ii. step response

%     ystep[n] = u[n] + u[n-1]

stepsignal = [ 1 1 1 1 1 1 1 1 ];

ystep = filter( [1 1], 1, stepsignal );

n = [ 0 1 2 3 4 5 6 7 ];

figure;

stem(n, ystep, 'LineWidth',2 );

ylim( [-0.5 2.5] );

xlim( [-0.5 7.5 ] );

% iii. Plot the magnitude and phase of

%      the frequency response using freqz.

%      H( exp(j w) ) = 1 + exp(-j w)

figure;

freqz( [ 1 1 ] );

% Magnitude response: Vertical axis is in

%   deciBels.  AdB = 20 log10(A). A linear

%   of a 1 equals 0 dB. In the magnitude

%   plot, low frequencies pass through

%   with gain (at or above 0 dB) and very

%   high frequencies (close to pi) are

%   attenuated.  Lowpass filter.

% Phase response: A line with a slope of

%   -1/2 if the phase were plotted in rad.