EE 338L CMOS Analog Integrated Circuit Design

Lecture 7, Current Mirrors/Sources

Current mirrors/sources are widely used in analog integrated circuits. They can work as

(i) Bias circuit, and/or,

(ii) Signal processing circuit,

such as, current source load for common source amplifiers, tail current source for differential pairs, bias currents for folded cascode amplifier.

The current sources can be copied from a reference master current source, which usually comes from a band-gap reference circuit.

How could we generate copies of currents from a reference current source?

The answer is to use – current mirrors!

The basically principles of current mirrors are very simple, as shown in the next page.

For a MOSFET, if ID=f(VGS), then VGS=f-1(ID), Iout=f[f-1(IREF)]=IREF. Here we assume M1 and M2 have the same size and VDS.

As VDS1=VGS, when VDS2=VGS, we have    VGS1=VGS2= VGS, VDS1=VDS2=VGS. If M1 and M2 have the same size, their drain currents are equal. (Why?)

Note that for a MOS transistor in saturation,

ID=12KPWL(VGS−VT)2(1+λVDS)

We have

ID1=12KPWL1(VGS−VT)2(1+λVDS1), and

ID2=12KPWL2(VGS−VT)2(1+λVDS2)

Thus,

IOUTIREF=ID2ID1=WL2WL11+λVDS21+λVDS1

Note that IOUT is well defined, mainly determined by the geometries of the transistors, and1+λVDS21+λVDS1. For simplicity, we assume two transistors have identical size. If two transistors have different VDS, the mirroring is not prefect. But usually λis small (such as 0.05 V-1), thus the error (say, a few percent) is usually tolerable. We will also study ways to improve the current mirror accuracy.

Note that the correct operation of current does not rely on square law. As low as theID=f(VGS) is monotonous, the current mirror will work. Thus we also have bipolar transistor current mirror as shown in right figure.

Example: Find the current of M4 if all transistors are in saturation (the geometry ratio of Mi is WLi).

Solution:

We haveID2=IREF⋅WL2WL1, ID3=ID2, and ID4=ID3⋅WL4WL3,

SoID4=α⋅β⋅IREF=WL2WL1⋅WL4WL3⋅IREF, where α=WL2WL1,β=WL4WL3

Proper choice of α and β can give large and small ratios between ID4 and IREF.

Example: Calculate the small-signal voltage gain of the circuits shown in right figure. Assume (W/L)3/(W/L)2=α, and all transistors are in saturation. Please ignore λ effect (λ=0). The transconductance of M1 is gm1.

Solution:

The small signal equivalent circuit is

Node equations

gm1vin+gm2vx=0   (1)

gm3vx+voutRL=0       (2)

From above equations, we can get

Av=voutvin=gm1gm3gm2RL=gm1RLWL3WL2

Current Mirror Error

Assume M1 and M2 are in the saturation.

ID1=12KPWL1(VGS−VT)2(1+λVDS1)

ID2=12KPWL2(VGS−VT)2(1+λVDS2)

So,     ID2ID1=(WL)2(WL)11+λVDS21+λVDS1

Note that VDS1=VGS, thus when VDS2=VGS, the current mirror is “accurate”.

Note for small channel transistors, VT of MOSFET changes with transistor size, especially L. To have better mirror accuracy, M1 and M2 should have the same L.

For current mirror ratio WL2WL1other than 1, multiple unit transistors should be used. Each unit transistor has the same W and L.

Cascode Current Mirrors

To suppress the error introduced by channel length modulation effect, cascode current mirrors can be used.

The key idea of cascode current mirror is to match the VDS of two bottom transistors, M1 and M2.

Cascode current mirror, note that (WL)M2(WL)M1=(WL)M3(WL)M0=α

Shown in the above Figure, M0 and M1 are diode connected MOS transistors. For M0 and M1, we have VGS0=VDS0, and VGS1=VDS1. The potential at the drain/gate of M1 is VGS1, and the potential at the drain/gate of M0 is VGS1+VGS0, as shown in Fig. 1.

Note that if we ignore channel length modulation effect, (WL)M2(WL)M1=(WL)M3(WL)M0=α, we have IOUT=IREF, and VGS3=VGS0, VGS2=VGS1.

If we consider channel length modulation effect,

1) if VO=VN, due to the symmetry, IOUT=IREF.

2) if VO changes from VN to a higher value, we have, VO>VN. Note that ID=12KpWLVGS−VT2(1+λVDS). If we assume IOUT does not change, IOUT=ID3=12KpWLVGS3−VT2(1+λVDS3), VDS3 will be larger because VO is higher compared to the case 1),  (1+λVDS3) is larger, to keep IOUT unchanged, VGS3−VT2 will be smaller, hence VGS3 will be smaller, thus VY (or VDS2) will increase as VN keeps constant. Note that IOUT=ID2=12KpWLVGS2−VT22(1+λVDS2), other terms keep unchanged, while (1+λVDS2)increases, thus IOUT will increase.

Note that because VDS2 (=-VGS3) is much smaller than VO, the IOUT of cascode current mirror is much smaller than the IOUT of non-cascode simple current mirror.

Output branch of the cascode current mirror

From the small signal model as shown in Fig. 2(b), according to KCL, we have

i21+i22+i23=i1			(1a)

i21=−gm3vy				(1b)

i22=gds3(vo−vy)			(1c)

(1d)

i1=gds2vy				(1e)

Combine and rearrange the above derivations, we have

vyvo=gds3gm3+gmb3+gds3+gds2		(2)

According to the definition of small signal analysis, ΔVYΔVO≈∂VY∂VO=vyvo, we have

ΔVYΔVO≈vyvo=gds3gm3+gmb3+gds3+gds2≈1gm3+gmb3gds3+1=1Ait+1	(3)

where Ait=gm3+gmb3gds3, which is the gain of the top transistor.

Eq. (3) shows that VY is much smaller than VO, or ΔVY≈1Ait+1ΔVO. Thus VY is very close to VX, even if VO is different from VN.

Example: In the following figure, assuming (WL)M1(WL)M2=(WL)M0(WL)M3, sketch Vx and Vy as a function of IREF. If IREF requires 0.5V to operate as a current source, what is the maximum value of IREF? Note that the VT of M1 and M2 is VT,M1 and the VT of M0 and M3 is VT,M0, the transconductance coefficient of M1 and M2 is KP.

Solution:

When VN reaches its maximum value, IREF reaches its maximum value.

Since M2 and M3 are properly ratioed with respect to M1 and M0, we have VY=VX≈2IREFKP(WL)1+VTH1. The behavior is plotted in the figure b.

We note that

VN=VGS0+VGS1=2IREFKPLW0+LW1+VTH0+VTH1

Thus,

VN,max=2IREF,maxKPLW0+LW1+VTH0+VTH1=VDD−0.5V

and hence

IREF,max=KP2(VDD−0.5V−VTH0−VTH1)2LW0+LW12

Low Voltage Cascode Current Mirrors

Previously discussed current mirrors have relatively large voltage drop.

The left side has a voltage drop of 2VGS (note that in reality, VGS1 and VGS0 are not necessarily equal).

The right side has a minimum voltage drop of VGS2+Vdsat3. Otherwise, M3 will enter triode region.

One way to reduce the voltage headroom requirement is shown in the following:

1) Remove M0 in the above figure.

2) Bias the gate terminal of M3 with a bias voltage Vcas, which is chosen to keep M2 and M3 in saturation with minimum voltage headroom requirement at the output terminal.

What is the value of Vcas in the right figure?

Vcas = VDS2+VGS3=Vdsat2+VGAP+VGS3

Note that in above figure, M1 and M2 have different VDS values, VDS1=VGS1, while, VDS2=Vdsat+VGAP, where VGAP is the voltage safe margin to keep M2 in saturation. Thus,

IOUTIREF=(WL)2(WL)11+λVDS21+λVDS1

The term 1+λVDS21+λVDS1 is not unity, as VDS1≠VDS2. Thus large current ratio error will result.

How to solve the VDS mismatch problem?

Key features:

1) Input voltage drop: VGS (half of the input voltage drop of the folded cascode current mirror in previous page)

2) Minimum output voltage: 2Vdsat+VGAP, where Vdsat is VGS-VT, and VGAP is VDS-Vdsat of bottom transistors.

3) Output resistance:

Rout=[1+(gm2n+1+gmb2n+1)ro2n+1]ro2n+ro2n+1≈(gm2n+1+gmb2n+1)ro2n+1ro2n

where n=1,2,3…

4) Very WIDELY used in modern analog integrated circuits. Must be grasped firmly!

How to generate VCAS

- Use additional current source and transistor

VGS,MCAS = VT+Vdsat0+Vdsat1+VGAP.

Example: If (W/L)0=(W/L)1=(W/L)2=(W/L)3=(W/L)T, and VGAP=0, what is the (W/L) of MCAS?

Solution:

Since (W/L)0=(W/L)1=(W/L)2=(W/L)3=(W/L)T, if we ignore the channel length modulation and body effect, we have VGS0 = VGS1= VGS2= VGS3= VGS,

Thus Vdsat0= Vdsat1= Vdsat2= Vdsat3= Vdsat

VGS,MCAS = VT+Vdsat0+Vdsat1+VGAP= VT+2Vdsat

12KPWLCAS(VGS,MCAS−VT)2=IREF=12KPWLT(VGS−VT)2

So WLCAS(2Vdsat)2=WLTVdsat2

WLCAS=14WLT

Example: If (W/L)0=(W/L)1=(W/L)2=(W/L)3=(W/L)T, and VGAP=12VdsatT, VdsatT is the Vdsat of M0 to M3, what is the (W/L) of MCAS?

Solution:

VGS,MCAS = VT+Vdsat0+Vdsat1+VGAP= VT+2.5VdsatT

12KPWLCAS(VGS,MCAS−VT)2=IREF=12KPWLT(VGS−VT)2

So WLCAS(2.5VdsatT)2=WLTVdsatT2

WLCAS=16.25WLT

- Use additional current source, transistors and resistor

VCAS=2VGS−IREF⋅Rb

If we choose Rb to have a voltage drop on Rb as VT-VGAP, we haveVCAS=2VGS−(VT−VGAP)=VGS+Vdsat+VGAP

Thus, Rb should be

Rb=VT−VGAPIREF

Example: In above figure, all transistors have a (W/L) of 16, KPN=100uA/V2, VTN=0.7V, IREF=50uA, what is the value of Rb to yield a VGAP of 180 mV? (Assume square law applies, andλ=γ=0)

Solution:

Rb=VTN−VGAPIREF=0.7V−0.18V50uA=10.4KΩ

- Use one additional resistor

Note that VCAS should be

VCAS= VT+2Vdsat+VGAP=VGS+Vdsat+VGAP= VGS+ VRb

Thus the voltage drop on Rb is

VRb= Vdsat+VGAP=IREFRb

We have

Rb=Vdsat+VGAPIREF

Example: In above figure, all transistors have a (W/L) of 16, KPN=100uA/V2, VTN=0.7V, IREF=50uA, what is the value of Rb to yield a VGAP of 180 mV? (Assume square law applies, and λ=γ=0)

Solution:

Vdsat=2IREFKPNWL=2×50uA100uAV2×16=0.25V

So Rb=Vdsat+VGAPIREF=0.25V+0.18V50uA=8.6KΩ

Current mirrors that process signals

Current mirrors could be used in circuitry that provides the DC bias current to the active circuits. They can also be actively involved in processing the signal. In the textbook, these current mirrors are called active current mirrors.