ECE 313 Lecture Notes

Lecture 5 on Sampling & Aliasing Part 3

Tuesday, September 22, 2026

By TA Alice Liu

Demonstration of audio effects

- Tremolo. The Smiths, "How Soon is Now?", opening guitar riff has a tremolo effect by guitarist Johnny Marr, with a second voice playing slide guitar. Tremolo is rapid change in volume to give a sensation of motion, and is rapidly varying amplitude modulation effect. "Songs that Changed Music: The Smiths - How Soon Is Now".

- Another Tremolo Effect: Spatial Audio.
Play an audio signal that ping-pongs between the left and right audio channels (speakers) using amplitude modulation.  When cosine is 1, the sine is zero, and vice-versa.  The cosine and sine signals have low frequencies such as 1 Hz or 5 Hz.

- Vibrato. Chris Isaak, "Wicked Games", an example of vibrato effort using a guitar whammy bar. Vibrato is a slowly varying frequency modulation effect.

- Delay. Stella, Reverb and Delay Explained, start at 1:11 mark. Delay controls when echoes arrive, whereas reverb consists of delays that are not controlled.

…

Logistics:

- Mini-Project #1 Due Wednesday by 11:59pm on the course Canvas page

- Homework #3 to be assigned, due Friday by 11:59pm

…

Review of Sampling

The Sampling Theorem says that if a continuous-time signal x(t) has no frequencies higher than fmax, then the signal can be reconstructed from its samples if the sampling rate c is chosen to be fs > 2 fmax.  This is a wonderful theoretical result upon which practice builds on.  When sampling at sampling rate fs, frequencies up to but not including ½ fs are captured.

Sampling and Reconstruction

- Continuous Time discussion

  - To properly recreate the signal (all its frequency components), sampling frequency, fs, must satisfy: fs > 2*fmax, where fmax is the highest frequency component in the signal

  - Alternatively: given a sampling frequency, fmax, we can reconstruct any signal where its highest frequency component satisfies: fmax<12fs

  - Otherwise, we will have aliasing

- Discrete Time discussion

  - Given a continuous time sampling frequency, fs, in cycles per second, and a continuous time signal with a maximum frequency component, f0, in cycles per second, the discrete time sampling frequency, w0, that satisfies the Sampling Theorem should be: w0=2πf0fs, in radians per sample, where -π≤w0≤π

  - Else, if the Sampling Theorem does NOT hold, then ALIASING occurs, and w0 must be adjusted!

- Aliasing Example (see Slides 5-9)

Aliasing

- Classic “Aliasing”

  - Cause: Sampling rate fs does not satisfy the Sampling Theorem

  - Results: After sampling, consider a frequency f0 that was captured correctly by sampling because fs > 2 f0.  However, there are also an infinite number of frequencies outside the range -½ fs < f < ½ fs that will alias to frequency f0 due to sampling.  These aliasing frequencies are of the form f0 + m fs where m is an integer.

- Another kind of aliasing: Folding

  - Cause: Sampling rate fs does not satisfy the Sampling Theorem

  - Results: After sampling, consider a frequency f0 that was captured correctly by sampling because fs > 2 f0.  However, there are also an infinite number of frequencies outside the range -½ fs < f < ½ fs that will alias to frequency f0 due to sampling.  These aliasing frequencies are of the form -f0 + m fs where m is an integer.

Reconstruction

- Ideal Reconstruction

  - Sampling satisfies Sampling Theorem

  - Not realistic

    - We can’t sample every possible point

  - Not practical

    - We don’t need every possible point, we can work off of fewer samples

      - Saves space, compute

- Interpolation between sampled points

  - Zero-order Hold

    - Pro: Simple implementation

    - Con: clear artifacts

    - Application: used in some designs for digital-to-analog converters

  - Linear

    - Pro: Pretty good approximation

    - Con: more work, still has artifacts

    - Application: used in speech and audio signals; causes blurring effects in images

  - Curve-fitting

    - Pro: smooth curve

    - Con: complex implementation

    - Application: commonly used when expanding an image and interpolating the pixel values for the new pixels (e.g. expanding a 512 x 512 image into a 2048 x 2048 iimage).