Tune-Up Tuesday #2 Version 2.0

% Copy this file into a Matlab script window, add your code and answers to the
% questions as Matlab comments, hit "Publish", and upload the resulting PDF file
% to this page for the tune-up assignment.  Please do not submit a link to a file
% but instead upload the file itself.   Late penalty: 2 points per minute late.

% (a) Copy, paste and run the Matlab code from lecture slide 1-18 to generate a
% cosine signal x(t) = cos(2 pi f0 t) with f0 = 440 Hz and play it as an audio signal
% for 3s at a sampling rate of fs = 8000 Hz.  440 Hz is an 'A' note on the fourth
% octave on the Western scale.  See Piano key frequencies.

% (a) ‘A’ note on Western scale in 4th octave (A4) at 440 Hz

f0 = 440;               % note ‘A4’

fs = 8000;              % sampling rate

Ts = 1/fs;              % sampling time

t = 0 : Ts : 3;         % 3 seconds

x = cos(2*pi*f0*t);

sound(x, fs);
pause(3);

% (b) Add to the code in (a) to generate a new signal  y(t) = cos(2 pi f0 t) cos(2 pi f1 t)
% with f1 = 110 Hz by using the same sampling rate of fs = 8000 Hz.  The code on
% the left side of lecture slide 3-3 might be helpful.  Please remember to use the .*
% operator for pointwise multiplication of two vectors.

% (b) Multiply cosine at 440 Hz and cosine at 110 Hz

% Modified code from left side of lecture slide 3-3.

f1 = 110;               % ‘A’ in the second octave

x1 = cos(2*pi*f1*t);

y = x .* x1;

% (c) Add to the code in (b) to play y(t) as an audio signal.
%       Describe what you hear.
%       Express y(t) as a sum of two sinusoids.

% (c) Play y(t) at a sampling (playback) rate of 8000 Hz.

%     I hear two notes/tones, one at a frequency lower than

%	440 Hz and one at a frequency higher than 440 Hz.

%	For me, x(t) and y(t) are about the same loudness,

%	and x(t) is more pleasant than y(t).

%

%     The product can be written as a sum of two cosines.

%     Using lecture slide 3-2,

%     y(t) = 0.5 cos(2 pi 330 t) + 0.5 cos(2 pi 550 t)

%     Tones at 330 Hz (E 4th octave) and 550 Hz (C# 5th octave)

%     are harmonics of 110 Hz. See Piano frequencies.

sound(y, fs);    % soundsc(y, fs) could also have been used.

% Note: The product cos(2*pi*f0*t) cos(2*pi*f1*t) will have

% amplitudes on [-1, 1] because each cosine has amplitudes

% on [-1, 1]. Hence, the sound command can be used without

% clipping amplitude values.

% (d) Plot one period of y(t).  We'll first need to find the period of y(t).
% The product of two sinusoids with frequencies f0 and f1 produces
% frequencies at f0+f1 and f0-f1.  You could modify the code from the
% bottom right side of lecture slide 3-3.

ffund = gcd(f0+f1, f0-f1); % 440 Hz +/- 110 Hz

Tfund = 1/ffund;

n = round(Tfund / Ts);     % Tfund / Ts isn’t an integer.

plot( t(1:n), y(1:n) );

% See below for the plot.  Although not asked, the plot has several
% artifacts as noted below.

Non-smooth troughs