Tune-Up Tuesday #3 Version 1.0

Written by: Alice Liu (TA)

% Tuneup Tuesday #3 for September 15, 2026

% Copy this file into a Matlab script window, add your code and answers to the

% questions as Matlab comments, hit "Publish", and upload the resulting PDF file

% to this page for the tune-up assignment.  Please do not submit a link to a file

% but instead upload the file itself.

% Late penalty: 2 points per minute late.

% The following code clears the Matlab workspace

clear all;         % clears all Matlab variables

close all;         % closes all figures

clc;               % clears the command window of text

% In this in-lecture assignment, we examine an audio effect, tremolo, onto

% a pure tone. We examine what happens when we apply amplitude modulation

% to a signal in the time domain and observe the effects in time and frequency.

%% Part 1

% Generate a cosine signal at frequency 440 Hz to play it as an audio

% signal at a sampling rate of 8000 Hz:

f0 = 440;       % 440 Hz (A in the 4th octave, or A4)

fs = 8000;      % sampling rate in Hz (a.k.a. samples per second)

Ts = 1/fs;      % sampling time in seconds

t  = 0 : Ts : 3;% sampling time, from 0s to 3s

x  = cos(2*pi*f0*t);

sound(x, fs);

pause(3);

%% Part 2.

% Generate and apply a pulse-shaping signal p(t) for amplitude modulation

%       p(t) = A*cos(2*pi*f_tau*t)

% You may choose any value of A and f_tau, and we recommend you try

% different values. Share the values of A and f_tau that you tested.

% (a) Try A = {0.1, 0.9} with f_tau = 0.5

% (b) Try f_tau = {0.5, 5} with A = 0.9

% (a1) A=0.1, f_tau = 0.5

A1  = 0.1;

f_tau1 = 0.5;

p1 = A1*cos(2*pi*f_tau1*t);

y1 = p1 .* x;

% (a2) A=0.9, f_tau = 0.5

A2  = 0.9;

f_tau2 = 0.5;

p2 = A2*cos(2*pi*f_tau2*t);

y2 = p2 .* x;

% (b1) A=0.9, f_tau = 0.5

A3  = 0.9;

f_tau3 = 0.5;

p3 = A3*cos(2*pi*f_tau3*t);

y3 = p3 .* x;

% (b2) A=0.9, f_tau = 5

A4  = 0.9;

f_tau4 = 5.0;

p4 = A4*cos(2*pi*f_tau4*t);

y4 = p4 .* x;

%% Part 3.

% Please play the audio signals in Part 2, and describe what you hear. How

% does the value of A and f_tau affect the audio signal you hear? (It may

% be helpful to play the initial signal, x, and compare against y.)

% Play a vector of amplitude values as an audio signal.

% Amplitude values less than -1 are clipped to -1.

% Amplitude values greater than 1 are clipped to 1.

% (a1) A=0.1, f_tau = 0.5

sound(y1,fs);

pause(3);

% (a2) A=0.9, f_tau = 0.5

sound(y2,fs);

pause(3);

% (b1) A=0.9, f_tau = 0.5

% (This is a repeat, it's ok if the repeat is not included in submission.)

sound(y3,fs);

pause(3);

% (b2) A=0.9, f_tau = 5

sound(y4,fs);

pause(3);

Discussion response for (3):

% Our initial signal, x, is a single tone at 440 Hz, with a maximum amplitude

% of 1. Compared to the initial signal, in our modified tremolo signals, we

% hear the initial signal, x, change in terms of volume (amplitude).

% We compare the audio signals of part (a), with constant f_tau and

% variable maximum signal amplitude, A.

% When A is small (A=0.1), the maximum volume of the signal is softer.

% When A is large (A=0.9), the maximum volume of the signal is louder.

% Since f_tau is the same, we hear the amplitude modulation occur at the

% same rate.

% We compare the audio signals of part (b), with constant maximum signal

% amplitude and variable f_tau.

% When f_tau is small (f_tau=0.5 Hz), we hear the volume change slowly.

% When f_tau is large (f_tau=5.0 Hz), we hear the volume change quickly.

% Since both f_tau values are relatively small compared to the initial

% frequency, f0, it sounds as though the tremolo signal is still at the

% same frequency of 440 Hz. Depending on how sensitive one's hearing is,

% one may either hear 2 frequencies, or just 1 frequency that sounds like

% the initial 440 Hz.

% Since the amplitude A is held constant, we should hear that the maximum

% volume of the signal remains the same.

% Food for thought (not graded for submission):

% When f_tau is very large (say, for f0 = 440 Hz, we choose f_tau = 110 Hz),

% we can start to distinguish 2 tones in the audio signal, like in the beat

% signals from Tune-up Tuesdays #2 last week. Thus, to ensure we create a

% tremolo effect, we must let f_tau remain relatively small when compared

% to the initial frequency, f0.

%% Part 4.

% Analyze the resulting tremolo signal in the time and frequency domain

% for f_tau = 5 with A = 0.9.

% Do the plots demonstrate the same behaviors you noticed in part 3?

% Why or why not?

% You may want to zoom in on the frequency domain and spectrogram

% plots for your analysis.

% Does the resulting tremolo signal share the same period with the

% original tone?

% For f_tau = 5, A = 0.9, this is option (b2), which is signal y4.

% Thus, we now plot the Time-domain analysis:

plot_timeDomainAnalysis(y4, fs);

% And the Frequency-domain and Spectrogram analysis:

plot_freqDomainAnalysis(y4, fs);

% Alternatively, we can specify the type of signal in the title of the plot:

plot_timeDomainAnalysis(y4, fs, "Tremolo");

plot_freqDomainAnalysis(y4, fs, "Tremolo");

%% Discussion response for (4):

% Yes, the time-domain plot supports our observations from (3).

% The time-domain plot demonstrates the amplitude modulation of the

% original signal. We see that over time, the amplitude of the pure tone

% decreases to 0 before it increases to 0.9. If we were to plot option (a1)

% we would see that the amplitude only increases to 0.1 instead.

% At a glance, the frequency-domain representation of the signal shows the

% frequencies appearing at f0=440 Hz. However, if we zoom into the

% frequency-domain representation of the signal, we will notice that the

% frequency components present appear at +/-(f0+f_tau) and +/-(f0-f_tau).

% This is in line with what we hear and with what we understand. Because

% the f_tau used is relatively small, we don't expect to hear an audible

% difference in the pitch of the signal. But we understand based on the way

% we have generated the tremolo signal, that the actual frequencies present

% must the combination of the two cosines in the equation,

% y = cos(2*pi*f_tau*t).*cos(2*pi*f0*t).

% We see the same 2 frequencies in the spectrogram as well, if we zoom in to

% around 440 Hz.

% The resulting tremolo signal for option (b), signal y4, should have a

% period of Tfund = 1/ffund = 1/gcd(f0+f_tau,f0-f_tau) = 0.2 seconds;

% We see that this is also the case when we examine the time-domain plot.

% Notice that at these timestamps (0.04875s and 0.24875s), the signal

% amplitude y4(t) are exactly the same:

% y4(t=0.04875s) == y4(0.24875s) == -0.0336045

xlim([0 0.3]);

Tfundamental = 0.24875s-0.04875s = 0.2s